AUTOMATED LOAD TRANSIENT TEST SCRIPT DOCUMENTATION & FAILURE ANALYSIS REPORT

Python Script overview

The PDN Load Transient Test Script is a Python-based automated test program that controls four bench instruments over USB using the PyVISA library and SCPI (Standard Commands for Programmable Instruments) protocol. It executes a defined load transient test sequence on each voltage rail of the Power Distribution Network, captures oscilloscope waveforms, extracts key transient metrics, logs all results to a CSV file and generates a structured test report.

1. Rails tested - +3V6 (2.5A), +3V3 (3A), +1V8 (3A), +2V5 (1.5A)

2. Captures per rail – 10 (Can be modified)

3. Outputs - Per-capture waveform CSVs, Results CSV and Text Report

Script Architecture

- Configuration block

This block basically consists of all the parameters that needs to be altered before running the script. It mainly includes the VISA address, rail definitions, acceptance criteria and scope settings. It also includes the board serial number.

- Instrumentation class

This block handles the connection, sends SCPI commands and reads responses. If a connection fails, it tells which instrument and the reason for failure.

- Setup functions

There are primarily 4 setup functions - setup_psu, setup_load, setup_scope_channel and setup_dmm. Each one sends the exact SCPI commands needed to configure that instrument for the test. These run once at the start.

- Waveform capture block

This function is used to configure the scope into single-trigger mode, execute the load step, wait for the capture and then download the waveform. It saves each capture as its own CSV file with timestamp in the filename.

- Transient Analysis function

Takes the raw waveform array and extracts all four measurements (Overshoot, Undershoot, Recovery time and settling time). Since the scope is AC coupled, the DC component is removed — so overshoot appears as a positive peak and undershoot as a negative trough above and below zero. Recovery and settling times are measured from the trigger point.

- Pass/Fail Evaluation

evaluate_pass_fail() compares each measurement against the acceptance criteria derived from the ±5% voltage tolerance specification. It returns a pass/fail verdict for each parameter individually, and an overall result that is PASS only if all four parameters pass.

- CSV logger

Every capture from every rail is written as one row to a CSV file.

CSV columns:

- Timestamp — date and time of that specific capture

- Board_SN — serial number of the board under test

- Rail — which rail was being tested (+3V6, +3V3, etc.)

- Capture_No — which capture number (1 through 10)

- Overshoot_mV, Undershoot_mV, Recovery_us, Settling_us — raw measured values

- OS_PF, US_PF, Rec_PF, Set_PF — per-parameter PASS/FAIL

- Overall_PF — PASS only if all four parameters pass

- Report Generator

generate_report() runs after all rails are tested. It reads the captured results and produces a structured text report containing: overall board result, per-rail pass/fail summary with capture count, acceptance limits for each rail, statistics table (min, max, mean, standard deviation) across all captures per rail, and a per-capture breakdown table showing every individual measurement.

Failure Analysis

The failure that has been observed here is that the Board SN – 017 has failed the load transient test on the 3.3V rail intermittently. It has failed 3 out of the 10 captures.

Steps to Analyse

Step 1: Re-run with more captures

Run for 50 captures instead of 10 captures. With only 10 captures and a 30% failure rate, the confidence interval is wide. 50 captures reduce statistical uncertainty and allows a reliable distribution plot. This helps in getting a better statistical picture. Then a distribution plot can be plotted to understand the failure more accurately.

Step 2: Identify which parameter is failing

From the CSV, isolate exactly which of the four measurements (overshoot, undershoot, recovery, settling) is causing the fail. This gives an idea as to which parameter to focus on.

Step 3: Compare the failed board with other passing boards

Compare the CSV data from some of the boards that passed all 10 captures. Compare their mean and standard deviation on the +3V3 rail against SN-017. This can help us understand if it is board specific issue, based on the statistical values.

Step 4: Visual and physical inspection under a microscope

Inspect output capacitors C35, C37, C39 on the +3V3 rail and the feedback resistors R35/R36, for any capacitors that appears not fully seated, or has a cold solder joint (dull, grainy surface).

Step 5: Measure the output capacitance

Measure the capacitance at the +3V3 output while the board is unpowered. Compare against expected value (3 × 22µF = 66µF, de-rated to roughly 30–40µF at operating voltage). If it is significantly lower, then one of the capacitors may be missing or defective.

Step 6: Swap the suspected components

If Step 4 or 5 identifies a suspect component, replace the suspect capacitor or reflow the solder joint. Re-run the full 10-capture test.

Recommendations

- Quarantine the board and mark it is as under review.

- Perform Steps 4–6 listed above. The most efficient path is to inspect capacitors under microscope, measure ESR on C35/C37/C39.

- If a defective capacitor or cold joint is found and reworked, re-run the full transient test. If it passes, then accept the board and document the rework.

- If the same components used in SN-017 are used in other boards, then it would be better to verify the capacitance of those boards as well just to see if it has marginally passed or not.

- If no physical root cause is identified after completing all steps, the board must be scrapped. An intermittent failure with unresolved root cause must not be shipped.